1.Device Placement & Configuration PBQ:

1. Understand the scenario and the picture

Look at the diagram:

- Building A = stacked desktops → these will be wired clients.

- Building B = mostly laptops/phones/tablet → these will be wireless clients.

- Buildings are 50 ft apart with no cable between them, so you must bridge the gap wirelessly.

- Requirements:

  - Both buildings must reach the Internet.

  - All Internet traffic must be inspected before entering the network.

  - Desktops should not see other devices’ traffic → you want switching, not hubs / shared collision domains.

On the picture you’ll see 5 dashed drop zones:

- dz1 – inside Building A.

- dz2 – high up in Building B (wireless area).

- dz3 – in the “core” path between Internet and buildings (top-middle).

- dz4 – in the vertical path between Internet cloud and the link toward B.

- dz5 – midway horizontally between the buildings.

You also have these devices in the list:

- Hub, Switch, WAP, Firewall, Router, Wireless range extender

Each device can only be used once.

2. Place the correct devices

Dropzone 1 (dz1 – inside Building A)

Goal: connect many desktops in A, keep their traffic segmented from each other.

- A hub would flood all traffic and violate “desktops should not see traffic destined for other devices.”

- A switch creates separate collision domains and uses MAC tables.

Drag Switch to dz1.
Reason: Gives wired connectivity and segmentation for the desktops in Building A.

Dropzone 2 (dz2 – Building B, near all wireless clients)

Goal: give wireless access to laptops/phones in B.

- That’s exactly what an Access Point (WAP) does.

Drag WAP to dz2.
Reason: Provides Wi-Fi for devices in Building B.

Dropzone 3 (dz3 – between Internet and internal network)

Goal: “Security insists that all Internet traffic be inspected before entering the network.”

- That is the job of a firewall (packet filtering, stateful inspection, etc.).

- It must sit between the Internet and the internal router so all inbound/outbound traffic flows through it.

Drag Firewall to dz3.
Reason: Ensures all Internet traffic is inspected before it reaches internal networks.

Dropzone 4 (dz4 – under the firewall, before traffic splits)

Goal: connect internal networks to each other and to the firewall.

- You need Layer-3 routing between:

  - Building A’s LAN

  - Building B’s LAN (wireless clients)

  - Possibly a shared “transit” network toward the firewall/ISP

- That is the job of a router.

Drag Router to dz4.
Reason: Routes between internal subnets and the firewall/Internet.

Dropzone 5 (dz5 – between the two buildings)

Goal: the buildings have no cable, but B needs to reach the same network/Internet.

- You already placed a WAP in Building B.

- To get signal from Building A side across the 50-ft gap, you use a wireless range extender (or bridge/repeater).

Drag Wireless range extender to dz5.
Reason: Extends the WAP’s signal across the gap so Building A or the core can reach Building B wirelessly.

What about the Hub?

- You should not place the Hub anywhere.

- If you place it, you’ll lose points because hubs don’t provide segmentation and go against “desktops should not see other devices’ traffic.”

3. Configure the WAP (dz2)

After placing the WAP, a tiny magnifying glass (🔍) appears in that drop zone.

- Click the 🔍 on the WAP to open WAP Settings.

- Some fields are locked (name, gateway, SSID, passphrase) – leave them as is.

- In the Wireless section:

  - Mode → set to AC
(5 GHz, high throughput, matches the answer key.)

  - Channel → set to 36
(A common 5 GHz channel; just use first channel available.)

- In Security Configuration:

  - Security Settings → choose WPA2 – Enterprise.

  - Passphrase field is read-only and already set to S3cretkey! – don’t change it.

Close the dialog when done.

You’ve now got:

- Modern 5 GHz AP (AC), on channel 36,

- Using strong WPA2-Enterprise security.

4. Configure the Wireless Range Extender (dz5)

Click the 🔍 on the Wireless range extender.

Goal: The extender must match the WAP’s wireless settings so it can repeat the same network.

- In the Wireless section:

  - Mode → set to AC.

  - Channel → set to 36.
(Must match the WAP’s channel.)

- In the Security section:

  - Security Settings → choose WPA2 – Enterprise.

  - Key/Passphrase → set to S3cretkey! (must match the WAP exactly).

Close the dialog.

Now the extender and WAP are effectively part of the same secure wireless network spanning the gap between the buildings.

5. Configure the Firewall (dz3)

Click the 🔍 on the Firewall.

You’ll see a simple rules table. Set the Action column like this:

- DNS Rule

  - Source: 192.168.0.1/24 → internal clients

  - Dest: ANY, Port 53
Action: PERMIT
(Internal hosts must be able to do DNS lookups.)

- HTTPS Outbound

  - Source: 192.169.0.1/24 (typo’d subnet in the sim, but treat as internal)

  - Port 443
Action: PERMIT
(Allow secure web browsing to the Internet.)

- Management (SSH)

  - Dest: 192.168.0.1/24, Port 22
Action: PERMIT
(Allow secure remote management.)

- HTTPS Inbound

  - Dest: internal subnet on port 443
Action: DENY
(Block Internet-originated HTTPS from reaching internal hosts—no public web server here.)

- HTTP Inbound

  - Dest: internal subnet on port 80
Action: DENY
(Block insecure HTTP inbound to internal hosts.)

Click Save (or close) when finished.

These rules satisfy: “Security insists that all Internet traffic be inspected before entering the network” and restrict unsolicited inbound connections.

6. Double-check the overall design

Before you hit Submit, mentally trace traffic:

- Client in Building B → WAP (dz2) → Extender (dz5) → Router (dz4) → Firewall (dz3) → Internet.

- Desktops in Building A → Switch (dz1) → Router → Firewall → Internet.

- Switch, router, and firewall give segmentation so:

  - Desktops don’t see each others’ traffic like they would on a hub.

  - Internet must pass through the firewall first.

Everything meets the three requirements.

7. Submit and interpret results

- Click Submit at the bottom.

- The Results dialog will pop up:

  - If you placed all devices correctly, configured WAP/Extender/Firewall as above, and did not use the Hub, you should score 17/17 and see a “Perfect!” message.

  - If not, read the feedback in the dialog; it will tell you:

    - Which device was misplaced,

    - Which setting (mode/channel/security/passphrase/firewall rule) is wrong,

    - Or if the hub was used.

You can click Reset All to scramble configurations and practice again.

2.Network Performance Simulation PBQ:

STEP 1 — Read the Scenario Carefully

You are told:

- There was a power outage.

- Users now report:

  - Slow access to application servers.

  - Wireless users have intermittent Internet problems.

So you are troubleshooting performance, not total loss.

STEP 2 — Click the FIRST TAB: “Device Health”

This loads the Device_Health.png screenshot:

Here you see:

- WAN Health Graph

- Latency

- Jitter

- Loss

- Throughput

- WAN1 vs WAN2 performance

STEP 3 — Look at WAN1 vs WAN2 for VoIP traffic

The question asks:

Which WAN station should be preferred for VoIP traffic?

To solve this, you must look at the VoIP-critical metrics:

VoIP Requires:

- Low latency

- Low jitter

- Low packet loss

🔍 From the chart:

| Metric | WAN1 | WAN2 | Which Is Better? |
| --- | --- | --- | --- |
| Latency | 24 ms | 11 ms | WAN2 |
| Jitter | 9.5 ms | 3.9 ms | WAN2 |
| Loss | 2.51% | 0.01% | WAN2 |

Correct Answer: WAN2

Why?
WAN2 has far superior:

- Lower latency

- Lower jitter

- Lower packet loss

This makes it ideal for VoIP, which must avoid choppy audio.

STEP 4 — Click the SECOND TAB: “Device Monitoring”

This loads the Device_Monitoring.png screenshot:

📎 Device Monitoring Image
(/mnt/data/Device_Monitoring.png)

This page contains:

- Device Status Donut Chart

- Top Hosts Table

These tell you what is down and what is generating huge traffic.

STEP 5 — Identify the Device With Connectivity Issues

The dropdown asks:

Which device is experiencing connectivity issues?

Look at the Device Status chart:

- 8 devices are UP (green)

- 2 devices are Warning (yellow)

- 3 devices are Alert (orange)

- 1 device is DOWN (red)

The only device in the simulation that aligns with the down condition from the chart is:

Correct Answer: WAP 2

Why?

- Wireless users are reporting problems.

- The chart shows one device DOWN.

- A wireless problem most likely points to WAP2 being offline, breaking wireless coverage.

STEP 6 — Identify the Workstation Generating the MOST Traffic

On the Top Hosts table:

| Rank | SRC Host | Bits |
| --- | --- | --- |
| 1 | 206.208.133.9 | 104.69 Gb |
| 2 | 10.1.90.53 | 80.93 Gb |
| 3 | 10.1.90.55 | 74.68 Gb |

Clearly the largest traffic source is:

Correct Answer: 206.208.133.9

Why?
It has 104.69 Gb — the highest traffic on the entire table.

This workstation could be:

- Backing up large files

- Compromised and exfiltrating data

- Running a bandwidth-intensive job

Either way, it’s consuming the most bandwidth.

STEP 7 — Submit Answers

Select:

Final Correct Selections

| Question | Correct Answer |
| --- | --- |
| WAN preference | WAN2 |
| Device with issues | WAP 2 |
| IP with MOST traffic | 206.208.133.9 |

Then click Submit.

If all are correct, you’ll get:

Score: 3/3 — All answers correct!

3. Network Troubleshooting PBQ:

STEP 1 — Understand the Scenario

The customer reports:

- Some PCs connect fine

- Some PCs cannot connect

- All devices should be in VLAN 10

Your job:

- Click each device

- Look at the cable test pattern

- Identify the problem

- Choose the correct solution

- Close dialog

- Repeat for all 9 devices

- Submit

STEP 2 — Know HOW to read cable test results

Each picture shows 1–8 pin pairs:

Normal Straight-Through Cable (Correct)

All vertical pairs aligned:

1 | 1

2 | 2

3 | 3

6 | 6

4 | 4

5 | 5

7 | 7

8 | 8

❌ Open Cable (one pair missing)

A vertical line disappears or is shorter.

❌ Short Cable (pair touches another)

Pins jump diagonally but only on one pair.

❌ Crossover Cable

Lots of X patterns crossing between wires.

❌ Connector Backwards

Pin order reversed or mirrored.

❌ Cable too long

Length > 100 meters.

❌ Wrong VLAN

VLAN does not match "VLAN 10" or device configured incorrectly.

STEP 3 — Evaluate Each Device Correctly

I’ll go device by device.

Device: SWITCH 1

- Cable shows straight-through

- VLAN shows VLAN 10

- Speed correct

- Length OK

Correct Problem: No Problem
Correct Solution: None

Device: SWITCH 2

- All pins correct

- VLAN correct

- No X patterns

- Length normal

Problem: No Problem
Solution: None

Device: SERVER 1

- Cable straight

- VLAN 10

- Length OK

Problem: No Problem
Solution: None

Device: PC1

- Straight-through

- VLAN 10

- Looks correct

Problem: No Problem
Solution: None

Device: PC2

Cable image:

1 2 | 1 2

3    | 3 6   <— 3 is OK, 6 missing on left (open)

4 5 | 4 5

7 8 | 7 8

- One line is short/missing

- This is a textbook Open Cable

Problem: Open cable detected
Solution: Replace cable

Device: PC3

- All lines correct

- VLAN 10

- Length normal

- Looks perfect

Problem: No Problem
Solution: None

Device: PC4

Identical wiring to PC2:

- Correct cable

- VLAN = 11 (should be 10)

Problem: Wrong VLAN
Solution: Change VLAN assignment

Device: PC5

The wiring looks like:

Pins are reversed on one end

- This is a textbook case of the connector being installed backward

- Pin order mirrored

Problem: Connector on backward
Solution: Replace cable

Device: PC6

- Straight-through

- VLAN 10

- Looks correct

Problem: No Problem
Solution: None

STEP 4 — FULL CORRECT ANSWER LIST FOR SUBMISSION

| Device | Problem | Solution |
| --- | --- | --- |
| Switch 1 | No Problem | — |
| Switch 2 | No Problem | — |
| Server | No Problem | — |
| PC1 | No Problem | — |
| PC2 | Open cable detected | Replace cable |
| PC3 | No Problem | — |
| PC4 | Wrong VLAN | Change VLAN assignment |
| PC5 | Connector on backward | Replace cable |
| PC6 | No Problem | — |

STEP 5 — Submit

After closing all dialogs, click Submit.

If all answers match the above, you get:

Score: 18/18
All answers correct!

4.Switch Port Configuration PBQ:

1. Read the VLAN key and the scenario

From the text + VLAN key:

- VLAN 10 – WLAN → APs / wireless SSIDs

- VLAN 20 – Server → server ports

- VLAN 80 – Wired → PCs, laptops (data)

- VLAN 100 – Printers → printers

- VLAN 160 – Voice → IP phones

Also:

- Disable unused ports.

- Set ports for redundancy where needed → that’s your LACP trunks.

2. Understand the port “roles”

Use the little gray labels under some ports:

- PC, Laptop → Wired VLAN 80

- Server → Server VLAN 20

- Printer → Printers VLAN 100

- AP → needs WLAN VLAN 10 (and usually data VLAN 80)

- IP_Phone → Voice VLAN 160 plus data VLAN for attached laptop

Watch the animated cables between switches: those are inter-switch links and should be trunk ports with LACP, carrying all VLANs.

So you will have three types of ports:

- Trunk LACP ports (uplinks/redundancy)

  - Enabled

  - LACP: ON

  - Speed: Auto

  - Duplex: Auto

  - VLANs: ALL (10,20,80,100,160)

- Access ports for end devices

  - Enabled

  - LACP: OFF

  - Speed/Duplex: Auto

  - VLANs: according to device type

- Unused ports

  - Disabled

  - No VLANs needed

3. Switch 1 – work each clickable port

Clickable: 2, 4, 18, 26, 29, 33, 38, 44

Ports 2 & 4 – uplinks to Switch 2

- You can see cables from Switch1 port 2 → Switch2 port 1 and Switch1 port 4 → Switch2 port 3.

- These are redundant uplinks → LACP trunks.

Set for both port 2 and port 4:

- Enable Port

- LACP Enabled

- Speed: Auto

- Duplex: Auto

- VLANs: 10, 20, 80, 100, 160 (all VLANs)

Ports 18 & 26 – PCs

Device labels say PC.

Set for port 18 and 26:

- Enable Port

- LACP Disabled

- Speed: Auto

- Duplex: Auto

- VLANs: 80 only (wired data)

Ports 29 & 33 – unused

- No device labels, no cables → must be shut down.

Set for port 29 and 33:

- Enable Port (unchecked / disabled)

Once you disable, VLANs don’t matter; leave everything else Auto/empty.

Ports 38 & 44 – servers

Device labels show Server.

Set for port 38 and 44:

- Enable Port

- LACP Disabled

- Speed: Auto

- Duplex: Auto

- VLANs: 20 only (server VLAN)

4. Switch 2 – core / aggregation switch

Clickable: 1, 3, 18, 28, 37, 39, 47, 48

Ports 1 & 3 – to Switch 1

- Cables from Switch2 1 & 3 back to Switch1 2 & 4.

- Redundant uplinks → trunks with LACP.

Set for port 1 and 3:

- Enable Port

- LACP Enabled

- Speed: Auto

- Duplex: Auto

- VLANs: all VLANs (10,20,80,100,160)

Ports 18 & 28 – servers

- Device labels show Server on Switch 2 too.

Set for port 18 and 28:

- Enable Port

- LACP Disabled

- Speed: Auto

- Duplex: Auto

- VLANs: 20 (server)

Ports 37 & 39 – trunks to Switch 3

- You see cables from Switch2 37 & 39 → Switch3 1 & 2.

Set for port 37 and 39:

- Enable Port

- LACP Enabled

- Speed: Auto

- Duplex: Auto

- VLANs: all VLANs

Ports 47 & 48 – trunks to Switch 4

- Cables from Switch2 47 & 48 → Switch4 1 & 2.

Set for port 47 and 48:

- Enable Port

- LACP Enabled

- Speed: Auto

- Duplex: Auto

- VLANs: all VLANs

5. Switch 3 – access for PCs, printer, AP+Laptop

Clickable: 1, 2, 18, 28, 34, 39, 43, 48

Ports 1 & 2 – uplinks from Switch 2

- These receive LACP trunk links from Switch2 37 & 39.

Set for port 1 and 2:

- Enable Port

- LACP Enabled

- Speed: Auto

- Duplex: Auto

- VLANs: all VLANs

Ports 18 & 28 – PCs

Labels show PC.

Set for port 18 and 28:

- Enable Port

- LACP Disabled

- Speed: Auto

- Duplex: Auto

- VLANs: 80 (wired)

Port 34 – printer

Label shows Printer.

Set for port 34:

- Enable Port

- LACP Disabled

- Speed: Auto

- Duplex: Auto

- VLANs: 100 (printers)

Ports 39 & 43 – unused

No device labels.

Set for port 39 and 43:

- Enable Port (disabled)

Port 48 – AP + Laptop

Label shows AP and LAPTOP on same port: AP providing WLAN plus a wired client.

So this port must carry:

- WLAN VLAN 10 (for wireless clients)

- Wired VLAN 80 (for laptop traffic)

Set for port 48:

- Enable Port

- LACP Disabled

- Speed: Auto

- Duplex: Auto

- VLANs: 10 and 80

6. Switch 4 – IP Phone, printer, PCs

Clickable: 1, 2, 18, 25, 32, 33, 42, 45

Ports 1 & 2 – uplinks from Switch 2

Connected to Switch2 47 & 48.

Set for port 1 and 2:

- Enable Port

- LACP Enabled

- Speed: Auto

- Duplex: Auto

- VLANs: all VLANs

Port 18 – IP phone + laptop + WLAN

Label: IP_PHONE and LAPTOP (and often this same jack feeds an AP or SSID).

This jack must carry:

- VLAN 160 – Voice for the IP phone

- VLAN 80 – Wired for laptop data

- VLAN 10 – WLAN if AP is hanging off it / SSID voice support

Set for port 18:

- Enable Port

- LACP Disabled

- Speed: Auto

- Duplex: Auto

- VLANs: 10, 80, 160

Port 32 – printer

Label: Printer.

Set for port 32:

- Enable Port

- LACP Disabled

- Speed: Auto

- Duplex: Auto

- VLANs: 100 (printers)

Port 42 – PC

Label: PC.

Set for port 42:

- Enable Port

- LACP Disabled

- Speed: Auto

- Duplex: Auto

- VLANs: 80

Ports 25, 33, 45 – unused

No device labels → shut them down.

Set for ports 25, 33, 45:

- Enable Port (disabled)

7. General config choices (same across all ports)

For every enabled port:

- Speed: Auto

- Duplex: Auto

LACP is only ON for the redundant uplink/trunk pairs between switches.

8. Submit and interpret results

- When all clickable ports are set:

  - Click Submit.

- Any port still misconfigured will turn red.

- Adjust only those red ports and re-submit until your score reads:

Score: 32 / 32 (or full for however many clickable ports the script counts).

5. Network Routing PBQ:

1. Understand the goal and the topology

Goal in the sidebar:

Validate routing between the network with Workstation A and the network with File Server 2.

From the diagram:

- Workstation A is in SITE B on subnet 10.0.1.0/24 (inside the larger block 10.0.0.0/22).

- File Server 2 is in SITE A on subnet 10.0.5.0/24.

- Routers and WAN links:

  - Router A (Site A)

  - Router C (middle)

  - Router B (Site B)

  - Router-to-router links:

    - A–C: 172.16.27.0/30

    - C–B: 172.16.27.4/30

So when Workstation A talks to File Server 2, traffic should go:

Workstation A → Router B → Router C → Router A → File Server 2

The whole exercise is: which router is missing a route and what route do we add?

2. Open Router A and analyze its routing table

- Click Router A on the picture → dialog opens.

- Make sure you’re on the Routing Table tab.

You see (important parts):

Router-A# show ip route

...

Gateway of last resort is 0.0.0.0 to network 0.0.0.0

S* 0.0.0.0/0 is directly connected, GigabitEthernet1

10.0.0.0/8 is variably subnetted, 4 subnets, 2 masks

C 10.0.4.0/22 is directly connected, GigabitEthernet2

C 10.0.6.0/24 is directly connected, GigabitEthernet2

...

C 172.16.27.0/30 is directly connected, GigabitEthernet3

Key observations:

- Router A has a default route 0.0.0.0/0 via Gi1 (toward Router C). That means anything not local goes to Router C.

- Router A directly knows 10.0.4.0/22 and 10.0.6.0/24 on Gi2 – this covers the Site A LANs (including 10.0.5.0/24 where File Server 2 lives).

- It also knows the WAN link 172.16.27.0/30.

So:

- A can reach Site B networks because it has a default route via C.

- A obviously knows its own Site A subnets, so there is no routing problem on Router A for our traffic path.

What you do in the dialog:

- Click the Routing Configuration tab.

- For “Was a problem found?”, select No.

- Don’t fill in any static route.

- Close the dialog.

3. Open Router C and analyze its routing table

- Click Router C → dialog opens.

- Look at the Routing Table tab:

Router-C# show ip route

...

10.0.0.0/8 is variably subnetted, 4 subnets, 2 masks

S 10.0.0.0/22 [1/0] via GigabitEthernet1

S 10.0.4.0/22 [1/0] via GigabitEthernet2

172.16.0.0/16 is variably subnetted, 2 subnets, 2 masks

C 172.16.27.0/30 is directly connected, GigabitEthernet2

C 172.16.27.4/30 is directly connected, GigabitEthernet1

What this means:

- 10.0.0.0/22 via Gi1 → “All of Site B’s LANs are reachable via Router B.”

- 10.0.4.0/22 via Gi2 → “All of Site A’s LANs are reachable via Router A.”

- It also knows both WAN /30 links as connected networks.

So Router C is acting as the hub and already has summary static routes to each site. That’s exactly what we want.

Conclusion: Router C has no problem.

In the dialog:

- Go to Routing Configuration.

- “Was a problem found?” → No.

- No static route needed.

- Close the dialog.

4. Open Router B and find the problem

- Click Router B → dialog opens.

- Check the Routing Table tab:

Router-B# show ip route

...

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 4 subnets, 2 masks

C 10.0.0.0/22 is directly connected, GigabitEthernet3

L 10.0.0.1/32 is directly connected, GigabitEthernet3

172.16.0.0/16 is variably subnetted, 2 subnets, 2 masks

C 172.16.27.4/30 is directly connected, GigabitEthernet1

L 172.16.27.5/32 is directly connected, GigabitEthernet1

Important points:

- Router B knows Site B LAN 10.0.0.0/22 (where Workstation A lives).

- It knows the WAN link 172.16.27.4/30 to Router C.

- There is no route at all to 10.0.4.0/22 (Site A networks).

- Also note: “Gateway of last resort is not set” → B has no default route either.

So when Workstation A sends traffic to File Server 2 (10.0.5.x):

- PC sends to default gateway = Router B.

- Router B looks at the routing table; there is no match for 10.0.5.x, so it drops the packet.

This is the broken spot.

We need to tell Router B: “If you need to reach the Site A networks (10.0.4.0/22), send them toward Router C via Gi1.”

That’s exactly the static summary route the answer key expects.

5. What is a summary (summarization) route and why 10.0.4.0/22?

Concept

A summary route (route summarization, route aggregation) is:

A single route that represents a group of more-specific networks.

Instead of advertising or configuring many /24 routes, we combine them into a bigger prefix, like a /22 or /21, as long as those networks are contiguous and share the same high-order bits.

Benefits:

- Smaller routing tables → less memory, faster lookups.

- Fewer routing updates → more stable network, less bandwidth on WAN links.

- Clearer design → one route per site instead of every little subnet.

Why 10.0.4.0/22?

In Site A we have multiple /24 networks that all live in the same block:

- 10.0.4.0/24

- 10.0.5.0/24 (File Server 2, DHCP2, RADIUS2)

- 10.0.6.0/24 (Workstations F & G)

- (Possibly 10.0.7.0/24 reserved for future)

A /22 covers exactly 4 /24 networks.

10.0.4.0/22 covers 10.0.4.0–10.0.7.255, which includes all of Site A’s LANs.

Instead of telling Router B:

- route 10.0.4.0/24 via C

- route 10.0.5.0/24 via C

- route 10.0.6.0/24 via C

…we just say “10.0.4.0/22 via C” and it covers everything.

6. Configure the correct summary static route on Router B

- With Router B’s dialog open, go to Routing Configuration.

- For “Was a problem found?” → select Yes.

- Fill in the static route fields using the answer key:

  - Destination Prefix: 10.0.4.0

  - Destination Prefix Mask: 255.255.252.0 (this is /22)

  - Interface: select Gi1 (GigabitEthernet1)

Why Gi1?

  - Gi1 is the interface from Router B toward Router C (172.16.27.4/30).

  - Router C already knows how to reach 10.0.4.0/22 via Router A, so B just needs to send all Site A traffic to C.

- Click Save Static Route.

The script adds a line like:

S 10.0.4.0 255.255.252.0 is directly connected, GigabitEthernet1

to the bottom of Router B’s routing table display.

- Optionally flip back to the Routing Table tab to confirm your new route appears.

8. Check your work with the “Check Score” button

- Click Check Score in the left sidebar.

- If everything matches, you’ll see:

  - Score: 3/3

  - Feedback lines:

    - Router A: Correct

    - Router B: Correct

    - Router C: Correct